STRUCTURAL ANALYSIS

By – Mehak Verma

Workflow :

- CAD Model Creation

- Designed a pulley model on Onshape CAD software with 5 grooves to represent the belt-contact geometry.

- The pulley was modelled with a central cylindrical hole for mounting/shaft support.

- Model Setup & Boundary Conditions

- Imported the pulley geometry in Altair Inspire for structural analysis ( .step model)

- Applied support to the inner cylindrical surface at the centre of the pulley to prevent rigid-body motion and restrict the degrees of freedom and represent the shaft constraint. (ie Displacement = 0)

- Application of Belt Forces

- Applied a 200 N tangential force on each groove-contact face.

- The forces were applied tangentially to represent the load transmitted by the belt through the pulley grooves.

- Structural Analysis

- Performed a linear structural analysis to determine the pulley’s response under the applied loading and constraints.

- Evaluated the resulting:

  - Total deformation/displacement

  - Tension and compression

  - Von Mises stress

Results :

- Total Displacement

- The maximum total displacement is approximately 1.498 × 10⁻⁶ m (1.50 μm), indicating very small overall movement of the pulley under the applied loading.

- The displacement is concentrated mainly around the loaded outer/groove region, while the central supported region shows minimal displacement.

- The small magnitude of displacement indicates that the pulley maintains its overall structural shape under the applied 200 N tangential forces.

- Deformation

- The deformation plot shows that the pulley undergoes limited deformation, with the highest deformation occurring toward the outer regions subjected to the applied belt forces.

- The central region remains relatively constrained due to the applied support at the inner cylindrical surface.

- Tension and compression

- The maximum tensile stress is approximately 8.319 MPa, while the maximum compressive stress is approximately 8.193 MPa.

- Tensile and compressive regions are distributed across the rim, spokes, and central hub, showing how the applied tangential forces are transferred through the pulley structure.

- The relatively similar magnitudes of tension and compression indicate a balanced stress response under the applied loading condition.

- Von-Misses Stress

- The maximum Von Mises stress obtained is approximately 8.319 MPa, with the highest values concentrated around the central hub/spoke transition and other geometric regions where stresses are concentrated.

- Most of the pulley remains in the lower-stress range, as indicated by the predominantly blue regions in the contour plot.

- The stress distribution shows that the pulley experiences localized stress concentration rather than widespread high stress under the applied loading condition.